Adapted from www.mediamanual.at, a communication platform of the          
Austrian Federal Ministry for Education, Arts and Culture


Microphone Types
 
The microphone is an energy converter, it converts sound energy (vibrations of the air) into electrical energy. This can be done through various devices and structures. The most important for our purposes are: the dynamic microphone and the condenser microphone.

The dynamic microphone

In dynamic microphones, the conversion of energy functions according to electromagnetic principles (coils and magnetic fields are involved, which may be familiar from physics class). Without going into technical details: what is important for practical applications? A dynamic microphone is always ready to use (it needs no battery) and is relatively insensitive, which has advantages and disadvantages.

The disadvantages: insensitive means that soft sounds are recorded poorly. 
The advantages: it is resistant to rough handling, to high humidity and wind; even very loud sounds do not lead to distortions in the microphone.

An interview is usually "loud" enough for a dynamic microphone, but for recording sounds it is usually less suitable.

Dynamic microphones usually do not sound as "neutral" as condenser microphones do (the term to describe this is "timbre"). This may, but need not necessarily be a disadvantage. Particularly in recording music, (e.g. pop and rock) special types of dynamic microphones are

often used for specific instruments. It is precisely this "colour" or timbre that best brings out the sound of these instruments or is particularly desirable in certain recording situations.

The condenser microphone

(also called electret condenser microphone). For energy conversion in such a microphone, a capacitor is involved (as the name suggests can be). No further technical details – just practical points:

A condenser microphone requires electric power, usually provided by a battery when used outside a studio.  In the studio, many condenser microphones can be supplied with power through the microphone cable, if the mixer and microphone are equipped to supply it. The most common procedure is a “phantom” power supply of 48 volts DC. The phantom power supply can normally be switched on or off at the mixing table. Condenser microphones that are built exclusively for studio use often can only run on phantom power supply (i.e., they will not take a battery). Some modern microphones are designed for both battery power (for mobile operation) and for phantom supply (when used in the studio).

If the microphone is left on, the battery will be empty after several days. Therefore, a condenser microphone must always be turned off when not in use. 
A condenser microphone is quite delicate, which has advantages and disadvantages.

Disadvantages: Very loud sounds can lead to distortions in the microphone, high humidity and rough handling can damage the microphone and even a light breeze can cause problems.

Advantages: Even quiet sounds and noises can be recorded. Condenser microphones generally sound more "neutral" than dynamic microphones. 

Conclusion: The condenser microphone can be used universally for radio work.

The Directional Pattern

This polar pattern provides an indication of the direction from which a microphone will best pick up sound.

The main directional characteristics are:

	omnidirectional
	cardioid
	super-kidney and club

All of the attributes described here (type of microphone, polar pattern, also mono/stereo) are independent of each other and can be combined almost at will.

Omnidirectional 
An omnidirectional microphone picks up sound from all directions. It can be used:

	only in very quiet rooms (e.g., in the studio)
	if several persons are to be recorded with only one microphone (meaning that they must
          speak at about the same distance from it)
	for capturing atmosphere

An omnidirectional microphone is not suitable for situations where there are environmental and background noises. Struggling with this is everyday routine for radio reporters. Therefore, omnidirectional microphones are less suitable for radio work, because they pick up surrounding noises.

Cardioid (uni-directional, cardioid) 
These microphones best pick up sound coming from the direction in which they are facing. The sound from the sides and rear will be more suppressed. This makes the ideal microphone for reporters and for radio work in general. It is very suitable for interviews and for many other recording situations, and should be used when there is disturbing background (it can be reduced, at least to some extent).

The microphone should be tilted in the direction from which the sound is coming. In an interview, the microphone should be constantly directed toward the interviewee: this helps to avoid missing the beginning of an answer due to waving the microphone back and forth from oneself to the interview guest.

Mics like the AKG C 1000 S or the Shure SM 58 cardioid are used often in radio work. Usually a symbol on the microphone (a kidney, for example) tells you about its directional pattern, otherwise the information is given in the technical documentation. However, the design of the microphone does not allow any conclusions about the directional characteristics. This is different with the following polar pattern.

Super-cardioid or club (hypercardioid)

Supercardioid and club microphones only perceive a very narrow range of the acoustic horizon. Here, it depends on the precise objectives.  Very often these microphones are used for film and video and in TV studios, where speakers are at a greater distance from the microphones. In radio, such microphones are rarely used. Possible application would be recording situations with a lot of background noise, such as takes on noisy streets.

Microphones with a super-cardioid polar pattern / lobe usually have an elongated shape and are therefore (unlike ball or cardioid) easy to recognize.

Mono and Stereo

Stereo recordings convey a spatial impression, mono recordings do not. For some situations, recording with a stereo microphone is better because it provides more "transparent" original sounds. Stereophonic sound is a wide field. Therefore, this issue can only be touched upon.

Today, most recording and playback devices are stereo equipment. That is, there are 2 channels which are processed separately (LEFT and RIGHT), to be heard separately out of two speakers. 

Mono recording: there are identical signals on both channels. 
Stereo recording: both channels have similar signals, which differ in details from one another. This creates a spatial impression for the listener. 

Stereo recordings can be made with 2 mono microphones, but this is too cumbersome for practical radio work. It is better to use one stereo microphone. Most stereo microphone have two small mono microphone inside their housing, which are slightly rotated around the axis of the microphone and thus "hear" to the left and right.

	A stereo microphone should not be rotated around its own axis during recording, since this may convert the left-and-right effect into up-and-down.

	Stereo recording requires more discipline and handling experience than mono recording.

	Interviews recorded in mono are usually easier to edit than stereo recordings.

The lavalier microphone

The lavalier microphone (derived from “pendant”) is usually clipped onto the clothing of the person speaking. It is often used in video or television work, seldom in radio. These are generally small condenser microphones with spherical or cardioid directional patterns.

Microphone Accessories

The windscreen is the main microphone accessory, a foam hood that is pulled over the microphone head. The porous shell allows the sound waves through, but reduces wind noise. This is not only of importance out of doors: in the studio, it protects against so-called pop noise occurring on consonants such as "p", "t" and "k". The windscreen can remain on the microphone at all times.

A tripod should be used only in the studio and in exceptional cases: in interviews, it is better to keep the microphone in hand. This makes it possible to respond to changes in the situation, such as when the interviewed person reclines or turns her/his head away from the microphone and suddenly begins speaking.

# Microphone Handling

The microphone never leaves the hand!

The proper way to hold the microphone is with a cable loop. Noise resulting from the movement of the cable in pivoting of the microphone will be reduced by the cable loop. The microphone should be held loosely in the hand, in a posture which can be maintained for some time without cramping – an interview usually lasts less than half an hour, but after 10 minutes even a lightweight microphone begins to feel very heavy.

Seat yourself so that the interviewee(s) can be achieved without contortions. It’s often practical to use a corner setting, in which the interviewer and the guest are sitting at right angles to one another. The distance from the microphone to the person speaking should be about 10 to 25 cm. 

Even if your own questions are spoken into the microphone, this distance should be respected. A common mistake: the interviewer holding the microphone much closer to him- or herself than to the interviewed person. As a result, the questions sound much louder and sometimes even distorted, while the answers are lacking in volume. 

The microphone is not pointed directly at the speaker’s mouth, but a bit below or from the side, to help avoid pop-noise. 

Always use headphones! 
This is the only way to judge what the recording really sounds like, to hear how much background noise is coming through and whether there are pop-sounds on consonants. The headphones should completely surround the ear and isolate well from the outside noise. "Earplug" phone as used on a walkman are therefore hardly suitable.